Consultant: __Tetra Tech          	Crew: CVB, MDH, RAV     WQ Meter: _YSI 556 MPS          Turbidity Meter: _MicroTPW      	QC1 Init, Date___ QC2 Init, Date___ QC3 LWZ, Date3/9/15

| Date Time | Location (Northing / Easting) | Location Description | Photo #s Description | Water Quality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Water Depth (ft) | Water Temp. (C) | Cond. (µS/cm) |  | Turbidity (NTU) | Comments |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Corr. | UnCorr. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/12/14 | 3182906.9 N 2102963.5 E | PRM 245.5 – inside of bed (R) single channel | 9443 – view u/s TRB 9444- view d/s TRB 9445 – view down bank TRB 9446 – view across xsec TRB - TLB | 1 | 9.92 | 101 | 142 | 162.7 | Sample site is d/s of high erosion site on glaciolacustrine near PRM 248 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/12/14 | 3165795.6 N 2080675.6 E | Oshetna TLB above confluence w/ Susitna | [No photos recorded] | 2 | 11.37 | 113 | 153 | 9.14 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1815 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/13/14 | 3167056.3 N 2071826.8 E | Upstream Goose Ck confluence – Susitna MC TLB | 9801 – view d/s from sample site 9802 – view u/s from samp site 9803 – view up Goose ck | 1 | 9.83 | 108 | 152 | 125.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1440 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/13/14 | 3167128.7 N 2071782.6 E | Goose Ck TRB | 9817 – view d/s fan TRB 9806 – view d/s Goose ck 9807 – view of Goose confluence | 1 | 10.43 | 79 | 57 | 1.16 | Cond. Corrected reading was with “C” on measuring inst. |  |  |  |  |  |  |  |  |  |  |  |  |
| 1455 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/13/14 | 3185288.9 N 2066896.9 E | Upstream UNT 228.5 confluence – Susitna MC TRB | 1050- view u/s UNT 228.5 1048- view d/s TLB from conf. 1047 – view u/s UNT 228.5 | 1.8 | 9.91 | 151 | 107 | 118.0 | “ |  |  |  |  |  |  |  |  |  |  |  |  |
| 1610 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/13/14 | 3185316.1 N 2066802.0 E | UNT 228.5 mouth | 1050- view u/s UNT 228.5 1048- view d/s TLB from conf. 1047 – view u/s UNT 228.5 | 1 | 9.86 | 199 | 141 | 0.84 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1625 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/14/14 | 3188082.7 N 2033425.7 E | Susitna PRM 221 TRB u/s trib | 1147 –view u/s, people at sample site | 1 | 8.85 | 151 | 105 | 135.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1530 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/14/14 | 3188191.4 N 2033354.2 E | TRIB at PRM 221 | 1151- view u/s 1152 – view d/s | 0.5 | 7.43 | 182 | 121 | 87.81 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1535 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/15/14 | 3212845.0 N 1993984.9 E | Jay Ck | 1258 – view u/s at Ck 1257 – view d/s at Ck | 1.0 | 7.39 | 204 | 135 | 2.83 | Conducted in lower most channel of Jay Ck. 3’ from TLB. 15 ‘ above confluce with Susitna. 5 cfs on probe |  |  |  |  |  |  |  |  |  |  |  |  |
| 1040 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/15/14 | 3212486.9 N 1994607.5 E | Susitna R. | 1279 – view u/s at site 1280 – view d/s at site | 1.3 | 9.17 | 150 | 107 | 136.1 | Conducted 40’ above beginning of Jay Ck. 6’ from TRB. 1 cfs on probe |  |  |  |  |  |  |  |  |  |  |  |  |
| 1100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/15/14 | 3214453.2 N 1985341.0 E | Susitna R. u/s Kosina Ck | [No Photos recorded] | 1.0 | 9.19 | 155 | 108 | 125.1 | Conducted u/s of upmost distributary of Kosina Ck |  |  |  |  |  |  |  |  |  |  |  |  |
| 1215 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/15/14 | 3214648.2 N 1985046.6 E | Kosina Ck | 1339, 1342 –view up Kosina 1343, 1340 – view d/s to mouth | 1.3 | 10.18 | 85 | 61 | 0.51 | Conducted 50’ u/s of mouth with Susitna |  |  |  |  |  |  |  |  |  |  |  |  |
| 1220 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/16/14 | 3230358.7 N 1931795.5 E | Watana Ck | 1407 – view u/s Watana 1409 – view d/s to mouth | 1.0 | 7.77 | 188 | 126 | 12.75 | Conducted 100’ u/s of mouth with Susitna. 4’ from TRB. 2 cfs on probe |  |  |  |  |  |  |  |  |  |  |  |  |
| 1122 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/16/14 | 3230253.5 N 1931948.4 E | Susitna R u/s Watana Ck | 1427 – view u/s on Su TRB 1428 – view d/s on Su TRB | 1.8 | 9.18 | 150 | 105 | 115.8 | Conducted u/s 50’ of above Watana ck. 3’ From TRB. 2 cfs of probe |  |  |  |  |  |  |  |  |  |  |  |  |
| 1150 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/16/14 | 3229523.2 N 1895706.3E | Susitna R. u/s Deadman Ck | 504 – view u/s TRB 505 – view d/s TRB | 1.5 | 9.55 | 148 | 105 | 125.6 | Conducted 250’ u/s above Dead man Ck mouth. 3’ from TRB. 1 cfs on probe |  |  |  |  |  |  |  |  |  |  |  |  |
| 1656 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/16/14 | 3229389.0 N 1895409.9 E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1700 |  | Deadman Ck. | 517 – view u/s 520 – view d/s ck. | 2.2 | 11.08 | 65 | 48 | 0.66 | Conducted 200’ u/s of confluence with Susitna. 2’ from TLB. 1 cfs on Probe |  |  |  |  |  |  |  |  |  |  |  |  |