Purpose: AL 1 is located at a scenic overlook of the Nenana River on the Denali Highway (MP XX). The purpose of this AL is to assess potential change in visual resources that may result from construction and operation of the transmission line ROW and/or potential road improvements on the Denali Corridor. View being analyzed is directed generally to the east. AL type is a Observation Point (OP), as the majority of viewers are assumed to be stopped at the lookout.

Landscape Character:

AL 1 is located in the Nenana River Basin LCT. Landscape is characterized by the broad, flat river basin, which appears large in scale.  Views are enclosed by the foothills of the Alaska Range to the north and the Chulitna Mountains to the south, limiting views to the foreground-middleground distance zone.  The  Denali Highway runs generally east-west, and appears as a straight to curvilinear line.

Dominant Character Elements:

Views from this location are dominated by the river valley and surrounding mountains, which direct attention generally to the north. Mountains appear conical to domed. River valley is broad and gently sloping and curvilinear. Vegetation is composed primarily of spruce, which appears stippled to dense in patches.  Isolated trees are apparent as narrow, conical, dark forms. Color appears black and white due to the contrast between the spruce forests and snow.

Distinct horizontal and diagonal lines are apparent. A strong horizontal line exists where the flat upland bench meets the river valley wall, created by the transition from dense, continuous upland spruce forests meet the less vegetation and snow covered valley walls. Likewise, distinct parallel lines are apparent in river valley walls due to strong contrast between dark, nearly black, vegetation against the snow covered landforms.  Bold, curvilinear lines exist in the river bottom, again due to strong contrast between snow and dark vegetation.  No structures are present, with the exception of interpretive sign and the Denali Highway.

Character Attributes

Landscape is characterized by moderate variety. Though mountains and

unity, and with overall intactness due to presence of water and surrounding landforms. The view appears common due to scale of mountains, and limited color variation in landscape.  View from from

View from AL to the southeast is small in scale and enclosed, largely due to the roadcut and foothill of the Chulitna Mountains in the foreground/middleground.